SOP for Equipment Update

- Update Process

- Software Prepare

  - Get relevant Software from William MO ( DHL CN)

  - Software list:

    - SP_Flash_Tool_exe_Windows_v5.1712.00.001.rar

    - DriverInstall.exe

    - w562a-20180625_en_telnet-image-fl

    - GoogleMobiControl1350_1677.apk

  - Unzip files in one folder

  - Install Patch “DriverInstall.exe”

- Burn in OS with private key

  - Open folder which involves all unzip files

  - Click and run “flash_tool.exe”

  - Choose loading file in unzip folder “MT6737M_Android_Scatter.txt”, software will automatically loading list.

  - Select burn in model to “Firmware Upgrade”

  - Click “Download” to start burn in, “download” button turn grey

  - Power off smart watch

  - Connect Smart Watch to the computer through USB, make sure smart watch have enough power to continue burn in activities.

  - When “Download OK” logo appears, burn in completed.

- Install SOTI Android Enterprise agent

  - Connect smart watch to computer through USB

  - Copy “GoogleMobiControl1350_1677.apk” to smart watch root directory

  - Running “File Manage” APP in smart watch, Select “ internal Storage”

  - Select correct APK to install agent, open “unknown sources” option to continue installation.

  - Installation completed, APP can be use until Step 5 completed.

- Device owner model setting

  - Enter smart watch setting  Security

  - Device administrators tick Mobicontrol

  - Enter Device owner tick mobicontrol

  - Smart watch will restart automatically

  - Mobicontrol can be use, update completed.

  - If Mobicontrol shows as below means device owner model setting failed, Reset device owner model setting follow Step 5 process.